Experiment – 6

Parseval's Relation and Harmonic Spectrum Analysis

(i) To verify Parseval’s relation for continuous-time periodic signals and analyze the
relationship between time-domain and frequency-domain power

Experimental Procedure

Generate the periodic signal over one period.

Compute time-domain average power.

Calculate Fourier coefficients up to N = 20.

Evaluate frequency-domain power using Parseval’s relation.

Compare the two values and calculate absolute error.

MATLAB Coding

clc; clear; close all;

%% 6(i) Verification of Parseval’s Relation for Periodic Signals

figure;

% Verification of Parseval's Relation

f0 = 1;

T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

% Time-domain average power

P_time = (1/T)*trapz(t,abs(x).^2);

% Fourier-series coefficients

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

% Frequency-domain power using Parseval's relation

P_freq = a0^2/4 + 0.5*sum(an.^2 + bn.^2);

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('Frequency-domain power = %.6f\n',P_freq);

fprintf('Absolute error = %.6e\n',abs(P_time-P_freq));

bar([P_time P_freq]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power'); grid on;

title('Verification of Parseval''s Relation');