Experiment – 6

Parseval's Relation and Harmonic Spectrum Analysis

(iv) To compare the total power obtained in the time domain with the harmonic power
distribution in the frequency domain.

Experimental Procedure

Generate the periodic signal and compute Fourier coefficients.

Calculate harmonic magnitude for n = 1 to 15.

Convert harmonic number to frequency.

Plot the harmonic spectrum using a stem plot.

Identify the significant components

MATLAB Coding

clc; clear; close all;

%% 6(iv) Comparison of Time-Domain Power with Frequency-Domain Harmonic Power

figure;

% Time-Domain and Frequency-Domain Power Comparison

f0 = 1; T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

P_time = (1/T)*trapz(t,x.^2);

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

P_dc = a0^2/4;

P_harmonics = 0.5*sum(an.^2+bn.^2);

P_frequency = P_dc + P_harmonics;

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('DC power = %.6f\n',P_dc);

fprintf('Harmonic power = %.6f\n',P_harmonics);

fprintf('Frequency-domain power = %.6f\n',P_frequency);

bar([P_time P_frequency]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power'); grid on;

title('Time-Domain vs Frequency-Domain Power');