Experiment – 8:

Laplace Transform, Inverse Transform, ROC and Stability

(ii) Determination of Inverse Laplace Transform for Signal Reconstruction

Experimental Procedure

Define X(s) symbolically.

Apply ilaplace to obtain x(t).

Evaluate x(t) over 0≤t≤5 s.

Plot the reconstructed time-domain signal.

Compare the plotted response with the analytical expression.

MATLAB Coding

clc; clear; close all;

%% 8(ii) Determination of Inverse Laplace Transform for Signal Reconstruction

figure;

% Inverse Laplace Transform

% Requires Symbolic Math Toolbox

syms s t

X = 1/((s+2)*(s+3));

x = ilaplace(X,s,t);

disp('Inverse Laplace Transform x(t) =');

pretty(x);

tt = 0:0.001:5;

xx = double(subs(x,t,tt));

plot(tt,xx,'LineWidth',1.2);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal Reconstructed using Inverse Laplace Transform');