Experiment – 7:

Continuous-Time Fourier Transform and Spectrum

(ii) To determine the magnitude and phase spectra of continuous-time signals and
analyze their significant spectral components.

Experimental Procedure

Define t and the sampling interval.

Generate x(t)=e^(−t²)cos(2π3t).

Compute the FFT-based transform with N = 4096.

Plot the signal, magnitude and unwrapped phase.

Identify the principal spectral peaks.

MATLAB Coding

clc; clear; close all;

%% 7(ii) Determination of Magnitude and Phase Spectra of Continuous-Time Signals

figure;

% Magnitude and Phase Spectrum of a CT Signal

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2).*cos(2*pi*3*t);

N = 4096;

X = fftshift(fft(x,N))*dt;

f = (-N/2:N/2-1)/(N*dt);

magX = abs(X);

phaseX = unwrap(angle(X));

subplot(3,1,1);

plot(t,x); grid on;

xlabel('Time (s)'); ylabel('Amplitude');

title('Continuous-Time Signal');

subplot(3,1,2);

plot(f,magX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)'); ylabel('|X(f)|');

title('Magnitude Spectrum');

subplot(3,1,3);

plot(f,phaseX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)'); ylabel('Phase (rad)');

title('Phase Spectrum');