Experiment – 7:

Continuous-Time Fourier Transform and Spectrum

(iii) Verification of Fourier Transform Properties and Frequency-Domain
Characteristics

Experimental Procedure

Generate x(t)=e^(−t²).

Create x(t−0.4) using interpolation.

Compute the FFT of the shifted signal.

Form the theoretical transform using the time-shift property.

Compare computed and theoretical magnitude and phase.

MATLAB Coding

clc; clear; close all;

%% 7(iii ) Verification of Fourier Transform Properties and Frequency-Domain

Characteristics

figure;

% Verification of Fourier Transform Properties

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2);

x_shift = interp1(t,x,t-0.4,'linear',0);

X = fftshift(fft(x))*dt;

N = length(x);

f = (-N/2:N/2-1)/(N*dt);

% Time-shift property: x(t-t0) <-> X(f)e^(-j2*pi*f*t0)

X_shift_theory = X.*exp(-1j*2*pi*f*0.4);

X_shift = fftshift(fft(x_shift))*dt;

subplot(2,1,1);

plot(f,abs(X_shift),f,abs(X_shift_theory),'--');

xlim([-5 5]); grid on;

xlabel('Frequency (Hz)'); ylabel('Magnitude');

title('Time-Shift Property: Magnitude Comparison');

legend('Computed','Theoretical');

subplot(2,1,2);

plot(f,unwrap(angle(X_shift)),f,unwrap(angle(X_shift_theory)),'--');

xlim([-5 5]); grid on;

xlabel('Frequency (Hz)'); ylabel('Phase (rad)');